PRODIF24

Hardware Documentation	release 29.05.1997

Features	2

Connection references	2

Configuration	2

Initialization	3

Control and status register 	4

Control register I	4

Control register II	5

Status register	6

Data transfer	7

Generally DMA Initialization	7

Audio format Initialization	7

Transfer	7

Organisation of the samples at the DMA-Buffer	7

Contact to the developers	9

# Features

The PRODIF24 soundcard includes optical and electrical connections for an external digital playback device and an external digital recording device. The digital formats AES/EBU and S/PDIF will be supported at the optical and the electrical connections. Additionally real time format converting will be supported by the PRODIF24. An analog output which a 18-Bit DA converter, allows to intercept the input or output signal through external analog devices.

The PRODIF24 supports the sample rates 32000 Hz, 44100 Hz and 48000 Hz. and recording/playback of    16-Bit and 24-Bit audio-material.

# Connection references

| Connection (up Þ down) | Function |
| --- | --- |
| Jack | Analog output |
| TOS-Link jack | Optical output |
| Jack | Electrical output |
| Jack | Electrical input |
| TOS-Link jack | Optical input |

# Configuration

For the configuration of the PRODIF24 the setting of an I/O-address is necessary. The setting is done through a seven-digit switch on the surface of the soundcard. The following schedule shows a selection of the most common I/O-addresses and its appropriate settings at the PRODIF24. The terms A3-A9 can be found again as an inscription at the printed circuit board (pcb). In fact A3-A9 represent the I/O-addresslines of the same name. Because of that the soundcard can be placed at an I/O-range of 000h-3F8h. The souncard needs an I/O range of    4 Bytes only

.

| Addr | A3 | A4 | A5 | A6 | A7 | A8 | A9 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 100h |  |  |  |  |  |  |  |
| 108h |  |  |  |  |  |  |  |
| 110h |  |  |  |  |  |  |  |
| 118h |  |  |  |  |  |  |  |
| 120h |  |  |  |  |  |  |  |
| 128h |  |  |  |  |  |  |  |
| 130h |  |  |  |  |  |  |  |
| 138h |  |  |  |  |  |  |  |
| 140h |  |  |  |  |  |  |  |
| 148h |  |  |  |  |  |  |  |
| 150h |  |  |  |  |  |  |  |
| 158h |  |  |  |  |  |  |  |
| 160h |  |  |  |  |  |  |  |
| 168h |  |  |  |  |  |  |  |
| 210h |  |  |  |  |  |  |  |
| 218h |  |  |  |  |  |  |  |
| 220h |  |  |  |  |  |  |  |
| 228h |  |  |  |  |  |  |  |
| 230h |  |  |  |  |  |  |  |
| 238h |  |  |  |  |  |  |  |
| 240h |  |  |  |  |  |  |  |
| 248h |  |  |  |  |  |  |  |
| 250h |  |  |  |  |  |  |  |
| 258h |  |  |  |  |  |  |  |

| Addr | A3 | A4 | A5 | A6 | A7 | A8 | A9 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 260h |  |  |  |  |  |  |  |
| 268h |  |  |  |  |  |  |  |
| 270h |  |  |  |  |  |  |  |
| 300h |  |  |  |  |  |  |  |
| 308h |  |  |  |  |  |  |  |
| 310h |  |  |  |  |  |  |  |
| 318h |  |  |  |  |  |  |  |

empty field:	switch off    (down)
	switch on    (up)

# Initialization

The PRODFI24 stores the greatest part of its control at a so-called FPGA-Chip. It will loose its information after a reset or if the computer is switched off. Because of that the Hardware has to be initialized before using it.

The FPGA-control is stored at the data file PRODIF24.RBT. The control informations have a line orientated ASCII format. The following Pascalroutine reads the information line by line from the data file f and writes it at the Buffer Data^.

function ReadConfigFile (Fln : string) : Boolean;

var

f : text;

s : string;

i : byte;

begin

Assign (f, Fln);

Reset (f);

if InOutRes = 0 then

begin

Cnt := 1;

while (InOutRes = 0) And not Eof (f) do

begin

Readln (f, s);

if InOutRes = 0 then

begin

if (s[1] = '0') or (s[1] = '1') then

begin

for i := 1 to Length(s) do

begin

Data^[Cnt] := s[i];

Inc (Cnt);

if Cnt > MaxSize then

begin

Writeln (#7, 'Data Buffer Overrun.');

Halt (1);

end;

end;

end;

end;

end;

Close (f);

end;

ReadConfigFile := IoResult = 0;

end;

After that the content of the buffer has to be converted and output at the I/O-Base address of the PRODIF24. This happens for example through the following Pascalroutine:

Data^	= 1-based Datapuffer array
Cnt	= number of the Bytes at the Datapuffer + 1
Base	= Base I/O-address of the card

procedure WriteConfigData;

var

i : LongInt;

begin

for i := 1 to Cnt-1 do

begin

if Data^[i] = '0' then

Port[Base] := $fe

else if Data^[i] = '1' then

Port[Base] := $ff

else

begin

Writeln (#7, 'Data Error.');

Halt (1);

end;

end;

end;

It is very important that the produced Bit stream will NOT be interrupted. For this reason all interrupts should be blocked before the start of this routine and released again after it. After the Download of the control information the hardware initialization has finished.

# Control and Status register

The PRODIF24 includes two control registers and one status register. The following schedule shows the registers position at the I/O-Addressrange of the card and its functionality.

| Register | Position | Access | Function |
| --- | --- | --- | --- |
| Control register I | BaseIoAdr | 16 Bit write only | Setting of the playback sample rate, recording/playback sample resolution (16 Bit, 24 Bit), Recording and playback DMA-channel |
| Control register II | BaseIoAdr+2 | 16 Bit write only | Setting of the recording input (optical, electrical), Switching the input to the output (Input-Monitoring, realtime format converting), Defining the digital format (AES/EBU, S/PDIF) |
| Status register | BaseIoAdr | 16 Bit read only | Contains informations about the input sample rate, input datastream errors and Input Channel Status |

The following schedules show the registers in detail and the meaning of the single bits.

## Control register I

| Bits | Function | Value | Comments |
| --- | --- | --- | --- |
| 0..1 | Playback sample rate | 00b Slavemode 01b 48000 Hz 10b 44100 Hz 11b 32000 Hz | Setting the playback sample rate influences the Baud rate of the digital data stream only. Additionally it is necessary to set the corresponding sample rate of the current digital format at the control register II, so that an external device can be locked at the correct sample rate. At Slavemode the digital transmitter works with the input sample rate always. Nevertheless the setting of the corresponding sample rate of the current digital format at the control register II and the "polling" of the input sample rate (notice Status register) is necessary. |
| 2 | Playback sample resolution | 0b 16 Bit 1b 24 Bit |  |
| 3 | Recording sample resolution | 0b 16 Bit 1b 24 Bit |  |
| 4..5 | Playback DMA-channel | 00b Disable 01b channel 5 10b channel 6 11b channel 7 | By setting a DMA-channel, a DMA-Request will be caused immediately. That means the DMA-Controller and the appropriate DMA-Buffer have to be initialized before the channel setting. |
| 6..7 | Recording DMA-channel | 00b Disable 01b channel 5 10b channel 6 11b channel 7 | By setting a DMA-channel and a digital signal present at the input a DMA-Request will be caused immediately. That means the DMA-Controller and the appropriate DMA-Buffer have to be initialized before the channel setting. |

## Control register II

| Bits | Function | Value | Comments |
| --- | --- | --- | --- |
| 0 | Digital format of the output will be chosen | 0b AES/EBU 1b S/PDIF |  |
| 1..3 | Reserved | 111b | These bits have to be set ALWAYS. |
| 4..5 | Playback sample rate if AES/EBU format has been set | 00b 32000 Hz 01b 44100 Hz 10b 48000 Hz 11b wenn S/PDIF | The chosen sample rate will be encoded by this Bits at the channel status of the output signal. The sample rates chosen at the control register I and II have to correspond always. If the format S/PDIF has been chosen the bits have to be set! |
| 6..7 | Playback sample rate if S/PDIF format has been set | 00b 32000 Hz 01b 44100 Hz 10b 48000 Hz 01b if AES/EBU | The chosen sample rate will be encoded by this Bits at the channel status of the output signal. The sample rates chosen at the control register I and II have to correspond always. If the format AES/EBU has been chosen, the bits have to include the value 01b! |
| 8 | Input will be chosen | 0b electrical 1b optical |  |
| 9 | Error Report/ Channel Status Informations | 0b Error Report 1b Channel Status | Not setting the bit leads to a input sample rate identification and a input error identification by the hardware. The informations can be picked up at the status register. If an error occurs it will be stored at the status register until the bit has been set for at least 2 µs. If the bit has been set the channel status infomations of the input can be picked up at the status register. |
| 10 | The channel for the Channel Status Informations will be chosen | 0b left channel 1b right channel | If Bit 9 has been set, it selects the channel, which channel status is represented at the status register |
| 11 | Input will be switched to the output | 0b not switched 1b switched | If the bit has been set, the audio data at the input will be output directly. Through Bit 0 the output format can be definded irrespective of the input signal. (realtime format converting) Depending on the chosen output format it has to be ensured, that the Bits 4..5 or 6..7 are set equivalent to the input sample rate (Status register) |

## Status register

The status register includes different informations depending on the status of control registers II    Bit 9:

Bit 9 of the control register II not set:

| Bits | Meaning | Value | Comment |
| --- | --- | --- | --- |
| 0..2 | Error information | 000b no error 001b Validity Bit High 010b Confidence Flag 011b Slipped Sample 100b CRC Error 101b Parity Error 110b Bi-Phase Coding Error 111b No Lock | An recognized error at the input data stream will be stored until Bit 9 has been set for at least 2 µs |
| 3..5 | Input sample rate | 000b Out of Range 001b 48 kHz ± 4% 010b 441. kHz ± 4% 011b 32 kHz ± 4% 100b 48 kHz ± 400 ppm 101b 44.1 kHz ± 400 ppm 111b 32 kHz ± 400 ppm |  |

Bit 9 of the control register II has been set, input format = AES/EBU:

| Bits | Meaning | Value | Comment |
| --- | --- | --- | --- |
| 0 | Digital format | 0b AES/EBU |  |
| 1 | Audio data | 0b Data are not Audio data 1b Audio data |  |
| 2..3 | Emphasis | 00b CCITT J.17 01b 50/15µs 10b not available 11b nothing coded |  |
| 4 | Unknown |  |  |
| 5 | CRC error | 0b CRC error recognized 1b no CRC errors | If there are errors, the Bits 1..4 should not be evaluated. |

Bit 9 of the control register II has been set, input format = S/PDIF:

| Bits | Meaning | Value |
| --- | --- | --- |
| 0 | Digital format | 1b S/PDIF |
| 1 | Audio data | 0b Data are not Audio data 1b Data are Audio data |
| 2 | Copybit | 0b copy forbidden 1b copy permitted |
| 3 | Emphasis | 0b 50/15µs 1b not available |
| 4 | Original | 0b Audio data are original 1b Audio data are not original. |
| 5 | Ignorant Category | 0b No attention paid to Copybit and original bit 1b Attention paid to Copybit and original bit |

# Data transfer

The audio data transfer from and to the PRODIF24 happens through a 16 Bit DMA. During one DMA-cycle one sample of the left and one sample of the right channel are transfered by the hardware. To ensure a continuos data transfer from and to the card the steps as shown below are necessary. Experiences with programming the DMA-controller or corresponding services of the operating system are assumed.

## Generally DMA Initialization

- Free 16 Bit DMA-channels have to be found out and stored

- Per DMA-channel max. 128 kByte physical(!) consecutively memory has to be reserved. For a sample resolution it is absolutly necessary that the seize of one DMA-Buffer is divisible through 4 (sample resolution of 16-bit) or divisible through 6 respectively (sample resolution of 24-bit)

- Recording and playback DMA channel have to be deactivated at the controller

- Address register, Counter register und Page register of the recording and playback channel have to be initialized through the appropriate buffer addresses and buffer sizes

- Recording DMA-channel has to be initialized at the operating mode Demand + AutoInc + AutoInit + Write

- Playback DMA-channel has to be initialized at the operating mode    Demand + AutoInc + AutoInit + Read

## Audio format Initialization

- Recording sample resolution has to be set at the control register

- Optical or electrical input has to be chosen at the control register II

- Playback sample resolution, -sample rate has to be set at the control register I

- Playback digital format and appropriate sample rates have to be set at the control register II

## Transfer

As the procedure for Record while Play shows, the activities for recording and playback at this example are interlaced. Of course recording and playback can be treated independent from each other too.

Start

- Playback sample data have to be copied at the playback DMA-Buffer

- Playback DMA-channel has to be activated at the controller

- Recording DMA-channel has to be activated at the controller

- Recording and playback DMA-channel have to be set at the control register I (Transfer starts)

Transfer loop

The following should be done cyclical, for instance timer controlled every 55 ms:

- Interrogate address register of the recording DMA-channel, pick up audio data if necessary

- Interrogate address register of the playback DMA-channel, refill audio data if necessary

Stop

- Recording and playback DMA-channel has to be set on zero at the control register I    (Transfer stops)

- Playback DMA-channel has to be deactivated at the controller

- Recording DMA-channel has to be deactivated at the controller

- Remaining recording audio data have to be picked up at the recording DMA-Buffer

- Address- und Counter register of the recording/playback DMA-channel have to be initialized again with the start values

## Organization of the samples at the DMA-Buffer

The following schedule shows, how the samples of the left and the right channel are organized at the DMA-Buffer. In detail how the content of the DMA-Buffer will be interpreted while transferring to the hardware and how the samples at the DMA-buffer are found by the user programm after recording.

Sample resolution 16 Bit:

| DMA_BufAdr | Bits 0..7 left channel |
| --- | --- |
| DMA_BufAdr+1 | Bits 8..15 left channel |
| DMA_BufAdr+2 | Bits 0..7 right channel |
| DMA_BufAdr+3 | Bits 8..15 right channel |

Sample resolution 24 Bit:

| DMA_BufAdr | Bits 8..15 left channel |
| --- | --- |
| DMA_BufAdr+1 | Bits 16..24 left channel |
| DMA_BufAdr+2 | Bits 8..15 right channel |
| DMA_BufAdr+3 | Bits 16..24 right channel |
| DMA_BufAdr+4 | Bits 0..7 left channel |
| DMA_BufAdr+5 | Bits 0..7 right channel |

# Contact to the developers

If you have problems or questions please do not hesitate to contact:

marian

Aribert Laschke & Andreas Hellner GbR
	Elektronik    und    Software    Entwicklung

Eisenacher Str. 72
	04155 Leipzig

Email:	marianGbR@t-online.de